安徽精能绿色能源有限公司

ATONE NEW ENERGY LIMITED

www.atoneenergy.comwww.atoneenergy.cn

2013.April./English Version

Solar Controller / Solar Inverter

The customer's demand is our greatest pursuit!

Enterprise profile：

Atone New Energy Limited is a focus on solar energy, wind energy, and other renewable power source product research and development, production, sales and service of high and new technology enterprise. , the main products are photovoltaic

Inverter and solar controller, the wind solar hybrid controller, and offer solar power system, solar street lamps system design, production and installation, debugging and other services.

The company has always been to the development of the enterprise technological innovation as power supply, with a high quality talent team, with renewable energy research and development of the power supply industry rich experience and strong independent innovation ability,and participate in government number of significant project.

Enterprise purpose:

Companies adhering to the "integrity, innovation, dedication, and cooperation" spirit of enterprise, to provide quality system design, installation, technique modification solutions services, and  continuously enhance customer and public satisfaction, strive to make the company more great contribution to society, global visibility of the new energy enterprise.

- Catalogue

- Catalogue ..............................................................................................................   1

- ★ Wind/solar hybrid controller series.................................................................   2

- 1. JW high-grade series wind solar hybrid controller..........................................    3

- 2. Economic wind solar hybrid controller.............................................................   6

- 3.JN-WD High-end type wind solar hybrid controller..........................................

- ★ Solar controller................................................................................................   7

- JC (5 A / 10 A) series solar controller ..............................................................   1

- JN-W（5A—10A）waterproof solar controller................................................  2

- JN-WS Solar Waterproof-Constant Current Integrated Machine Controller.....  4

- JW Series Solar Waterproof-Electricity Hybrid Controller................................  5

- JW Series Solar Electricity Hybrid Controller...................................................  6

- 6.. JN-T(20A-30A) series street light controller ....................................................  7

- 7. JC (30 A / 50 A) series controller.......................................................................   8

- 8. JN-S(5A-20A) series solar controller...............................................................   10

- 9. JC (100 A / 200 A) series high power solar controller......................................   11

- ★ Solar power system and the street lamp system .............................................   12

- 1. Solar power system ...........................................................................................  12

- 2. Solar street lamps system ..................................................................................  13

- 3. Wind solar hybrid street lamp system ...............................................................  14

- 4. Solar garden light system ..................................................................................  15

- 5. Solar lawn lamp system ....................................................................................  15

- ★ Inverter series ..................................................................................................  16

- AN-H series high frequency sine wave inverter................................................  16

- 1 AN-H300 series high frequency sine wave inverter .........................................  17

- 2. AN-H500 series high frequency sine wave inverter............................................  18

- 3. AN-H600 series high frequency sine wave inverter ...........................................  19

- 4. AN-H800 series high frequency sine wave inverter ...........................................  20

- 5. AN-H1000 series sine-wave inversion ...............................................................  21

- 6. AN-H1500 series high frequency sine wave inverter .........................................  22

- 7. AN-H2000series high frequency sine wave inverter...........................................  23

- 8. AN-H2500series high frequency sine wave inverter...........................................  24

- 9. AN-H3000 series high frequency sine wave inverter .........................................  25

- 10. AN-H4000 series high frequency sine wave inverter .......................................  26

- 11. AN-H5000 series high frequency sine wave inverter .......................................  27

- 12. AN-H6000 series high frequency sine wave inverter .......................................  28

- The company addresses and contact way: ................................................................  29

- ★ Wind solar hybrid controller series

Wind solar hybrid controller make full use of green clean energy, zero emissions, zero pollution, zero power consumption etc, and the products are widely used in the remote area with no electric power communication stations, radio and television, the ships, the family uses electricity, etc.

AW series controller can adapt to the hybrid scenery is very bad working environment, use electronic brake and electronic unloading, and other functions, solve the output of the wind power generator with the wind change and wide fluctuations problem, prevent to open load fan windmills accidents. Comprehensive consideration of the solar and wind power generation characteristics of different,the optimum combination of the system and the operation.

Product features:

1) Intelligent design, simple structure, control powerful, stable performance, safety and reliability of the products.

2) Should choose MPPT fan charging ways, charging efficiency than ordinary PWM way more superior

3) booster charge function, solve the charging efficiency, low because of fan speed low of difficult problems

4) Choose a big size LCD display, users all parameters directly visible

A human nature of the key operation, all of the parameters according to the request of users can adjust for customers in various environment in use

5) Choose a PWM stepless uninstall way.

6) Should the output state has a variety of functions, often power/half power/in different period of customization features, etc

7) liquid crystal display battery charging electric current and voltage

8) utility the complementary function, uninterrupted soft handoff, guarantee load to continue to work. (this function should be customized)

9) digital intelligent control.

◆perfect protection function:

| Protection function |  |  |
| --- | --- | --- |
| Protection function | Illustrate | note |
| Solar counter- attack | in the night and filling bad light, the battery voltage may be higher voltage across the solar array. The controller with a counter-attack charging road, in order to prevent the battery of solar cell produce reverse filling. |  |
| Battery reverse connect battery protection | at the battery, the controller does not work, no display. | Properly connected to battery anode can be normal operation |
| Open after long-term use battery protection | storage battery may happen to open or contact undesirable. This controller in the battery will protect the equipment itself after open without damage. | Regular inspection line connectivity |
| Overvoltage, wind speed and overload protection | in the wind or overvoltage state, the controller will automatically activated brake function to protect the fan and batteries. In an overload situation controller cut off the circuit automatically. | Circuit can be used maintenance can restore the resistance |

- AW series Booster type wind solar hybrid controller(MPPT charge)

1). Product picture, wiring picture

2). Technical parameters :

| Model |  | AW1230 | AW1240 | AW2430 | AW2440 | AW2450 | AW2460 | AW2480 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated voltage |  | 12V | 12V | 24V | 24V | 24V | 24V | 24V |
| Component power |  | 200W | 200W | 150W | 150W | 250W | 300W | 400W |
| Power of fan |  | 300W | 400W | 300W | 400W | 500W | 600W | 800W |
| charging | charging | 30A | 35A | 20A | 20A | 23A | 37.5A | 50A |
|  | Even charge protection | 14.4V±1% |  | 28.8V±1% |  |  |  |  |
|  | floating Charge | 13.8V±1% |  | 27.6V±1% |  |  |  |  |
|  | Even charge recover | 13.2V±1% |  | 26.4V±1% |  |  |  |  |
|  | Temperature Compensation | -24mV/℃ |  | -48mV/℃ |  |  |  |  |
| Over Discharge | Shut off (DC) | 10.8V±1% |  | 21.8V±1% |  |  |  |  |
|  | Resume (DC) | 12.3V±1% |  | 24.6V±1% |  |  |  |  |
| Over Voltage | Shut off (DC) | 16V±1% |  | 32V±1% |  |  |  |  |
|  | Resume (DC) | 15V±1% |  | 30V±1% |  |  |  |  |
| No-load Current |  | ≤0.1A |  | ≤0.1A |  |  |  |  |
| Voltage drop (DC) |  | ≤0.5V |  |  |  |  |  |  |
| Control mode |  | Fan MPPT charge function, PWM uninstall function, PWM over-current limiting |  |  |  |  |  |  |
| Boost charge |  | Flexible independent step-up circuit (optional) |  |  |  |  |  |  |
| Display |  | LCD |  |  |  |  |  |  |
| Show parameters |  | voltage、charge current，battery volumetric |  |  |  |  |  |  |
| Protected type |  | Lightning protection, solar cells, anti-anti-charge protection, battery open-circuit protection, battery reversal protection, over speed and over-voltage soft automatic brake protection, overload protection. |  |  |  |  |  |  |
| Heat emanation way |  | Radiator |  |  |  |  |  |  |
| Working Temperature |  | -25℃~+55℃ |  |  |  |  |  |  |
| Height |  | ≤5500m(2000m above the need to reduce power use) |  |  |  |  |  |  |
| Environment humidity |  | 0~90%，No condensation |  |  |  |  |  |  |
| Net Weight |  | 2.20KG |  |  |  |  |  |  |
| Size(L*W*H) |  | 164×181×100(MM) |  |  |  |  |  |  |

- 2.ANW series Economic type wind solar hybrid controller(PWM charge)

- 1).Product picture

- 2).Technical parameter

| Model |  | ANW 1230 | ANW 1240 | ANW 2430 | ANW 2440 | ANW 2440 | ANW 2450 | ANW 2460 | ANW 2480 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System rated voltage |  | 12V | 12V | 24V |  |  |  |  |  |
| Photovoltaic module power |  | 200W | 200W | 150W | 250W | 300W | 250W | 300W | 400W |
| Wind power generator power |  | 300W | 400W | 300W | 400W | 400W | 500W | 600W | 800W |
| Charging | Charging | 30A | 30A | 20A | 23A | 37.5A | 23A | 37.5A | 50A |
|  | Are filling protection | 14.4V±1% |  | 28.8V±1% |  |  |  |  |  |
|  | Float filling | 13.8V±1% |  | 27.6V±1% |  |  |  |  |  |
|  | Are filling recovery | 13.2V±1% |  | 26.4V±1% |  |  |  |  |  |
|  | Temperature compensation | -24mV/℃ |  | -48mV/℃ |  |  |  |  |  |
| Discharge | Disconnect (dc) | 10.8V±1% |  | 21.8V±1% |  |  |  |  |  |
|  | Recovery (dc) | 12.3V±1% |  | 24.6V±1% |  |  |  |  |  |
| Over- voltage | Cut off (dc) | 16V±1% |  | 32V±1% |  |  |  |  |  |
|  | Recovery (dc) | 15V±1% |  | 30V±1% |  |  |  |  |  |
| Idle current (dc) |  | ≤0.1A |  |  |  |  |  |  |  |
| Voltage drop (dc) |  | ≤0.5V |  |  |  |  |  |  |  |
| Control mode |  | Fan MPPT charge function, PWM unload function, PWM flow current limiting function |  |  |  |  |  |  |  |
| Booster charging |  | Flexible independent booster circuit (optional) |  |  |  |  |  |  |  |
| Display parameters |  | Voltage, charging electric current, storage battery power |  |  |  |  |  |  |  |
| Showing the way |  | LCD |  |  |  |  |  |  |  |
| Protection type |  | Lightning protection, Solar reverse charge protection, battery open-circuit protection, battery reverse protection, over wind speed and over-voltage soft automatic brake protection, overload protection etc. |  |  |  |  |  |  |  |
| Cooling mode |  | Radiator |  |  |  |  |  |  |  |
| Environment temperature |  | -25℃~+55℃ |  |  |  |  |  |  |  |
| Use altitude |  | ≤5500 m (2000 m above the need to reduce power use) |  |  |  |  |  |  |  |
| Environment humidity |  | 0~90%, not knot de |  |  |  |  |  |  |  |
| Weight |  | 1.15KG |  |  |  |  |  |  |  |
| Shape dimension |  | 165*140*64mm |  |  |  |  |  |  |  |

- AN-WD High-end type wind solar hybrid controller(Built-in unloading box and PWM charge)

- 1).Product picture

- 2).Technical parameter

| Model No. | AN-WD24 | AN-WD48 | AN-WD96 | AN-WD120 | AN-WD240 |
| --- | --- | --- | --- | --- | --- |
| Wind turbine power | 1kw | 1kw 2kw 3kw 5kw | 2kw 3kw 5kw 10kw | 3kw 5kw 10kw | 3kw 5kw 10kw |
| Solar max power | 300w | 300w 600w 1000w 1500w | 600w 1000w 1500w 3000w | 1000w 1500w 3000w | 1000w 1500w 3000w |
| Over-charge voltage | 28.8v | 57.6v | 116v | 144v | 288v |
| Over-charge recovery voltage | 26v | 56v | 110v | 138v | 264v |
| Method of control | PWM |  |  |  |  |
| Free-load lost power | ≤6mA |  |  |  |  |
| Voltage lost on charging | ≤0.26V |  |  |  |  |
| Voltage lost on discharge | ≤0.15V |  |  |  |  |
| Environment temperature | -25℃ - +55℃ |  |  |  |  |
| Shape Dimension | 440mm×366mm×166mm |  |  |  |  |
| Dump-load Dimension | 400mm×230mm×290mm |  |  |  |  |
| Net Weight | 8KG |  |  |  |  |
| Cooling Way | Cooling Fan |  |  |  |  |

★  Solar controller

Solar street lamps controller is a through the microprocessor control and display the intelligent charging and discharging controller, the high quality, low loss, the life is long components, to ensure the reliable performance, to make the street lamp system can long normal work, reduce the system maintenance cost, especially the maintenance of batteries to significantly reduce the cost.

AC series solar street lamps controller can adapt to bad work environment, can provide two loaded access by setting the function can be key to set up and free time set up light work time, according to the strength of the light automatic open or closed load, and can be used in low pressure LED, sodium lamp, energy-saving lamp, metal halide lamp, street lamp, garden lamp series of lighting system.

Product feature:

1.MCU control,achieve charge and discharge intelligent control.

2.With perfect memory function, the controller can according to the interrupt mode of work in front,when the controller re-energized

3.Time and light control, you can set any value at ten minutes 15 hours, realization of timing accuracy fine.

4.It has perfect battery over charge and discharge protection function.

5.Battery open-circuit,over voltage,output short-circuits protection function.

6.Bollard cell converse connection,converse charge protection at night.

7.Under voltage protection,shock protection,low no-load wasting.

8.Temperature compensation.

- 1. AC（5A/10A）series solar controller

1). Product picture

2). Technical parameters

| The product model | Discharge parameters (V) |  | Pressure parameters (V) |  | Charging parameter (V) |  | Way to work | State instructions | No-load Current (A) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Discharge protection | Discharge recovery | Over- voltage protection | Over- voltage reply | Are filling protection | Are filling recovery |  |  |  |
| AC1205LT | 11.1V±1% | 13.2V±1% | 16.5V±1% | 15.0V±1% | 14.4V±1%1 | 13.3V±1% | Double time Electric + control | Digital tube | ≤0.01 |
| AC1210LT | 11.1V±1% | 13.2V±1% | 16.5V±1% | 15.0V±1% | 14.4V±1% | 13.3V±1% | Double time Electric + control | Digital tube | ≤0.01 |
| AC2405LT | 22.2V±1% | 26.4V±1% | 33.0V±1% | 30.0V±1% | 28.8V±1% | 26.6V±1% | Double time Electric + control | Digital tube | ≤0.01 |
| AC2410LT | 22.2V±1% | 26.4V±1% | 33.0V±1% | 30.0V±1% | 28.8V±1% | 26.6V±1% | Double time Electric + control | Digital tube | ≤0.01 |

2. AN-W（5A—15A）waterproof solar controller

1).Product picture, Dimension figure:

2). Product features

(1). IP68 waterproof level, aluminum shell design, can effectively prevent all kinds of corrosion.

(2). 12 V / 24 V voltage to be automatic identification system.

(3). LED digital display and waterproof the key operate, use simple and quick.

(4). Improved tasting charging algorithm, every week for a balanced battery charging, effective

Prevent battery not balanced and curing phenomenon, improve battery service life.

(5). Five kind of load mode, convenient use in all kinds of street lamp and monitoring equipment.

(6). External temperature sensor, and has high precision temperature compensation.

(7) Parameters set off electricity save function without repeating Settings, convenience and fast.

(8) All kinds of state instructions.

(9). With the charger, discharge, overload protection and electronic short circuit protection and meter protector against fraud protection.

(10) TVS lightning protection.

| System voltage | 12V/24V Auto |
| --- | --- |
| System current | 5A/10A/15A |
| No-load loss | < 5mA |
| Solar input voltage | < 55V |
| Over-voltage protection | 16.5V；×2/24V |
| Balance charge voltage | 14.6V；× 2/24 V (25 ℃), maintain time: 1 hour |
| Ascension recharging voltage | 14.4V；2/24 V (25 ℃), maintain time: two hours |
| Float charging pressure | 13.8V；×2/24V （25℃） |
| Charging to return to voltage | 13.2V；×2/24V （25℃） |
| Discharge to return to voltage | 12.5V；×2/24V |
| Discharge protection | 10.8V；×2/24V |
| Temperature compensation | -4.0mv/℃/2V； |
| Electric voltage | Electric drive 7 V, electric close 2 V |
| Electric tell time | 1min |
| Overload, short circuit protection | 1.25 times rated current 3 minutes to be automatic recovery; 1.5 times rated current 5 minutes to be automatic recovery; ≥2 times rated current frequency, short circuit protection, could not be restored, need to manually check the recovery |
| Working temperature | -35 ℃ to + 65 ℃; |
| Protection grade | IP68 |
| weight | 0.15Kg |
| size | 65 x 67 x 18 (mm) (long x width × high) |

- 3. AN-WS Solar Waterproof-Constant Current Integrated Machine Controller

1).Product picture, Dimension figure:

2). Technical parameters

| System voltage | 12V/24V Auto |
| --- | --- |
| System current | 5A/10A |
| No-load consumption | < 5mA |
| Solar input voltage | < 55V |
| Over voltage protection | 16.5V; *2/24V |
| Equalizing charge voltage | 14.6V; *2/24V(25℃), hold time：1 hour |
| Float charge voltage | 13.8V; *2/24V (25℃) |
| Charge return voltage | 13.2V; *2/24V (25℃) |
| Over discharge voltage | 12.5V; *2/24V |
| Over discharge protection | 10.8V; *2/24V |
| Light control voltage | Light control OPEN 2V; *2/24V, Light control CLOSE 7V; *2/24V |
| Working temperature | -35℃-------+65℃; |
| Protection level | IP68 |
| Net weight | 0.15Kg |
| Size | 102*81*19.6(mm)（L×W×H） |

- AW Series Solar Waterproof-Electricity Hybrid Controller

1).Product picture, Dimension figure:

2). Technical parameters

| System voltage | 12V/24V Auto |
| --- | --- |
| System current | 5A/10A |
| No-load consumption | < 5mA |
| Solar input voltage | < 55V |
| Over voltage protection | 16.5V; *2/24V |
| Equalizing charge voltage | 14.6V; *2/24V(25℃), hold time: 1 hour |
| Improve charging voltage | 14.6V; *2/24V(25℃), hold time: 1 hour |
| Float charge voltage | 13.8V；*2/24V (25℃) |
| Charge return voltage | 13.2V；*2/24V (25℃) |
| Over discharge voltage | 12.5V; *2/24V |
| Over discharge protection | 10.8V; *2/24V |
| Light control voltage | Light control OPEN 2V; *2/24V, Light control CLOSE 7V; *2/24V |
| Working temperature | -35℃-------+65℃； |
| Protection level | IP68 |
| Switch voltage | 11.0; *2/24V |
| Net weight | 0.15Kg |
| Size | 84×81×18(mm)（L×W×H） |

- AW Series Solar Electricity Hybrid Controller

1). Product picture，dimension figure

2). Technical parameters

| Model |  |  | AC2405 |  | AC2410 | AC1205 | AC1210 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rated voltage |  |  | 24V |  |  | 12V |  |
| Max input power |  |  | 120W | 240W |  | 60W | 120W |
| current | Discharge |  | 5A | 10A |  | 5A | 10A |
|  | Charge |  | 5A | 10A |  | 5A | 10A |
| Charge | Average charge |  | 28.8V±1% |  |  | 14.4V±1% |  |
|  | Recover |  | 26.6V±1% |  |  | 13.3V±1% |  |
|  | Floating |  | (27.0V-27.6V)±1% |  |  | (13.5V-13.8V)±1% |  |
|  | Temperature Compensator |  | -36mV/℃ |  |  | -18mV/℃ |  |
| Start voltage |  |  | 24.6V±1% |  |  | 12.3±1% |  |
| Over discharge |  | Switching off | 22.2V±1% |  |  | 11.1V±1% |  |
|  |  | Recover | 26.4V±1% |  |  | 13.2V±1% |  |
| Over voltage |  | Switching off | 33V±1% |  |  | 16.5V±1% |  |
|  |  | Switching off | 30V±1% |  |  | 15V±1% |  |
| Non-load current |  |  | ≤10mA |  |  | ≤10mA |  |
| Max open circuit voltage |  |  | 50V |  |  | 25V |  |
| Voltage drop |  |  | Input ≤0.7V |  |  | Input ≤0.6V |  |
|  |  |  | Output ≤0.3V |  |  | Output ≤0.3V |  |
| Display |  |  | LED |  |  |  |  |
| Operating temperature |  |  | -25℃~+55℃ |  |  |  |  |
| Using altitude |  |  | ≤5500m(above 2000m need reduced the power ) |  |  |  |  |
| Product size and weight |  |  | 120mmx80mmx25mm 0.13Kg |  |  |  |  |

- 6.AN-T(20A-30A)series street light controller

1). Product picture，dimension figure

2).Technical parameters

| Rated voltage | 12 v, 24 v auto |
| --- | --- |
| Strong charging pressure | 14.8V/29.6V |
| Equilibrium charging | 14.5V/29V |
| Float charging pressure | 13.7V/27.4V |
| Under voltage protection | 10.8V/21.6 |
| Under recover voltage | 13V/26V |
| Maximum charging current | 30A |
| Maximum load current | 30A |
| Over load protection | 3 s recover 1.2 times, 10 s recover 1.5 times , more than two times need manual recover |
| Maximum wire diameter | 4mm² |
| Working temperature | -40-60℃ |
| Weight (gross) | 0.3KG |
| Product size | 136*92*31mm |
| Protection level | IP56 |

- 7. AC(30A/50A) series controller

1). Product picture

- .Technical parameters:

| The product model | Discharge parameters (V) |  | Pressure parameters (V) |  | Charging parameter (V) |  | Way to work | State instructions | Idle current (A) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Discharge protection | Discharge recovery | Over- voltage protection | Over- voltage reply | Are filling protection | Are filling recovery |  |  |  |
| AC1230LT | 11.1V±1% | 13.2V±1% | 16.5V±1% | 15.0V±1% | 14.4V±1% | 13.3V±1% | Double time Electric + control | LCD | ≤0.01 |
| AC1250LT | 11.1V±1% | 13.2V±1% | 16.5V±1% | 15.0V±1% | 14.4V±1% | 13.3V±1% | Double time Electric + control | LCD | ≤0.01 |
| AC2430LT | 22.2V±1% | 26.4V±1% | 33.0V±1% | 30.0V±1% | 28.8V±1% | 26.6V±1% | Double time Electric + control | LCD | ≤0.01 |
| AC2450LT | 22.2V±1% | 26.4V±1% | 33.0V±1% | 30.0V±1% | 28.8V±1% | 26.6V±1% | Double time Electric + control | LCD | ≤0.01 |
| AC4830LT | 44.4V±1% | 52.8V±1% | 66.0V±1% | 60.0V±1% | 57.6V±1% | 53.2V±1% | Double time Electric + control | LCD | ≤0.01 |
| AC4850LT | 44.4V±1% | 52.8V±1% | 66.0V±1% | 60.0V±1% | 57.6V±1% | 53.2V±1% | Double time Electric + control | LCD | ≤0.01 |

- 8. New Style AN-S(5A-20A) 12V/24V Auto Solar Controller

- 1). Product picture，dimension figure

2. Technology parameters:

| Rated voltage | 12 v, 24 v auto |
| --- | --- |
| Strong charging pressure | 14.8V/29.6V |
| Equilibrium charging | 14.5V/29V |
| Float charging pressure | 13.7V/27.4V |
| Under voltage protection | 10.8V/21.6 |
| Under recover voltage | 13V/26V |
| Maximum charging current | 20A |
| Maximum load current | 20A |
| Over load protection | 3 s recover 1.2 times, 10 s recover 1.5 times , more than two times need manual recover |
| Maximum wire diameter | 4mm² |
| Working temperature | -40-60℃ |
| Weight (gross) | 0.15KG |
| Product size | 120*78*35mm |
| Protection level | IP56 |

- 9. AC(100A/200A) series high power solar controller

- 1)．Product picture

- 2. Technical parameters:

| model | AND24V-100A | AND48V-100A | AND96V-100A | AND120V-100A |
| --- | --- | --- | --- | --- |
| Solar energy input current rating | 100A |  |  |  |
| Battery rated voltage | 24V | 48V | 96V | 120V |
| Owe pressure (V) | 21 | 42 | 84 | 105 |
| Owe pressure recovery (V) | 24 | 48 | 96 | 120 |
| Over-voltage (V) | 29.4 | 58.8 | 117.6 | 147 |
| Pressure recovery (V) | 28 | 56 | 112 | 140 |
| Float charging pressure (V) | 27 | 54 | 108 | 135 |
| Static loss | ≦0.09 |  |  |  |
| Showing the way | Liquid crystal display |  |  |  |
| Display parameters | Battery voltage, charging electric current, charging |  |  |  |
| Way to work | Continuous |  |  |  |
| Use of the environment | Temperature-10-+ 40 ℃ ℃, relative humidity is not more than 85% |  |  |  |
| Size (long * wide* high) | 41*25*16（cm） |  |  |  |
| Net weight (KG) | 6.8KG |  |  |  |
| Gross (KG) | 7.1KG |  |  |  |

★ Solar power system and the street lamp system

1. Solar power generation system

Product example:

solar cell module:1600W, free maintenance battery:800AH, the pure single inverter：3000W,charging controller：50A 。

Equipment can use appliances as well as the time is as follows:

| Design load electrical name | specifications | Power consumption power | number | Day of working time | Power consumption |
| --- | --- | --- | --- | --- | --- |
| Lighting | Energy-saving lamps | 11W | 8 | 6 | 528Wh |
| Computer | Liquid crystal display | 100W | 2 | 5 | 1000 Wh |
| Printer | laser | 250W | 1 | 1 | 250 Wh |
| Fax machine | inkjet | 150W | 1 | 1 | 150 Wh |
| Refrigerators | 150L | 100W | 1 | 24 | 800 Wh |
| Washing machine |  | 300W | 1 | 0.5 | 150 Wh |
| VCD Satellite TV receiver/VCD |  | 25W | 1 | 6 | 150 Wh |
| Color TV | 21 inches | 95W | 1 | 6 | 570 Wh |
| Pump |  | 400W | 1 | 1 | 400 Wh |
| Total |  | 1431W |  |  | 3998 Wh |

- Solar street lamp system

Product example:

| 35W solar low voltage sodium lamp system configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parts name | specifications | Unit | Quantity | Note |
| Solar panels | 17.6V/120W | pc | 1 | Single/crystal |
| Solar controller | 12V/10A | set | 1 | Electric+control |
| Battery | 12V/120AH | pc | 1 | Lead -acid/ colloid |
| LED light | 12V/35W | set | 1 | Orange |
| Street lamps |  | pc | 1 | Casting aluminum |
| The lamp pole and accessories |  | set | 1 | Hot dip galvanized coating |
| Above the system configuration can use 3 consecutive rainy days. According to different areas and different climate need to adjust. |  |  |  |  |

JNL-001                          JNL-002                        JNL-003

JNL-004                         JNL-005                         JNL-006

- 3．Wind/solar complementary street lamp system

- Product example:

| 35W solar low voltage sodium lamp system configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parts name | specifications | Unit | Quantity | Note |
| Solar panels | 17.6V/120W | pc | 1 | Single/crystal |
| Wind generator | 24V/300W | Set | 1 | Permanent magnetism generators |
| Solar controller | 12V/10A | Set | 1 | Electric+control |
| Battery | 12V/120AH | pc | 1 | Lead -acid/ colloid |
| LED light | 12V/35W | set | 1 | Orange |
| Street lamps |  | pc | 1 | Casting aluminum |
| The lamp pole and accessories |  | set | 1 | Hot dip galvanized coating |
| Above the system configuration can use continuously in 3 rainy days. According to different areas and different climate need to adjust. |  |  |  |  |

JNW-001                      JNW-002                      JNW-003

JNW-004                      JNW-005                     JNW-006

- Solar garden light system

JNG-001                        JNG-002               JNG-003               JNG-004

JNG-005                 JNG-006                   JNG-007              JNG-008

- 5.Solar lawn lamp system

JNN-001                        JNN-002                JNN-003               JNN-004

- JNN-005                   JNN-006                   JNN-007              JNN-008

★Inverter series

AE-H series high frequency sine wave inverter

AE-H series intelligent pure sine wave with high frequency inverter power supply PWM control technology and high power mosfet or IGBT components, so that the work products, low noise, fast response time output waveform good, working voltage and frequency stability, reliability, etc. This product has the automation of high automation, stable performance, the function is all ready, perfect protection, simple operation and other characteristics, can 24 hours work unattended, applicability, practical strong, used in the remote area with no electric power communication stations, television broadcasts, the ships, the family of electricity.

Product features:

1) The use of advanced CPU single-chip microcomputer intelligent control technology double, high reliability, low failure rate of characteristics.

2) Pure sine wave output, with load capacity, wide application.

3) With perfect protection function (voltage protection, over-voltage protection, output short circuit protection, thermal protection, overload protection).

4) Cooling fan intelligent control, the cooling fan CPU control of the work of the state, greatly extend the service life of the fan, and saving energy efficiency.

5) Work little noise, high efficiency, the output of the perfect pure waveform, prolong the service life of electrical equipment.

6) Low voltage alarm function.

- AE-H300 High frequency pure sine wave inverter

1) Product picture:

- Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 300W |  |  |
|  | SURGE POWER | 600W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 150*130*52mm |  |  |
|  | INNER BOX DIMENSION | 255*190*90mm |  |  |
|  | CARTON DIMENSION | 550*430*340mm |  |  |
|  | NET WEIGHT | 0.85±0.1KG |  |  |
|  | GROSS WEIGHT | 16±1KG |  |  |
|  | PACKING | 12PCS/CTN |  |  |

- AE-H500 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 500W |  |  |
|  | SURGE POWER | 1000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 210*150*70mm |  |  |
|  | INNER BOX DIMENSION | 325*190*110mm |  |  |
|  | CARTON DIMENSION | 430*350*440mm |  |  |
|  | NET WEIGHT | 1.4±0.2KG |  |  |
|  | GROSS WEIGHT | 14.5±1KG |  |  |
|  | PACKING | 6PCS/CTN |  |  |

- AE-H600 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 600W |  |  |
|  | SURGE POWER | 1200W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 210*150*70mm |  |  |
|  | INNER BOX DIMENSION | 325*190*110mm |  |  |
|  | CARTON DIMENSION | 430*350*440mm |  |  |
|  | NET WEIGHT | 1.8±0.2KG |  |  |
|  | GROSS WEIGHT | 14.6±1KG |  |  |
|  | PACKING | 6PCS/CTN |  |  |

- AE-H800 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 800W |  |  |
|  | SURGE POWER | 1600W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 310*150*70mm |  |  |
|  | INNER BOX DIMENSION | 390*190*110mm |  |  |
|  | CARTON DIMENSION | 440*420*320mm |  |  |
|  | NET WEIGHT | 2.6±0.2KG |  |  |
|  | GROSS WEIGHT | 16±1KG |  |  |
|  | PACKING | 4PCS/CTN |  |  |

- AE-H1000 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 1000W |  |  |
|  | SURGE POWER | 2000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 310*150*70mm |  |  |
|  | INNER BOX DIMENSION | 390*190*110mm |  |  |
|  | CARTON DIMENSION | 440*420*320mm |  |  |
|  | NET WEIGHT | 3.5±0.2KG |  |  |
|  | GROSS WEIGHT | 14.8±1KG |  |  |
|  | PACKING | 4PCS/CTN |  |  |

- AE-H1500 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 1500W |  |  |
|  | SURGE POWER | 3000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 340*150*70mm |  |  |
|  | INNER BOX DIMENSION | 455*190*110mm |  |  |
|  | CARTON DIMENSION | 490*430*300mm |  |  |
|  | NET WEIGHT | 4±0.5KG |  |  |
|  | GROSS WEIGHT | 17.2±1KG |  |  |
|  | PACKING | 4PCS/CTN |  |  |

- AE-H2000 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 2000W |  |  |
|  | SURGE POWER | 4000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 395*150*70mm |  |  |
|  | INNER BOX DIMENSION | 510*190*110mm |  |  |
|  | CARTON DIMENSION | 540*340*310mm |  |  |
|  | NET WEIGHT | 5.5±0.5KG |  |  |
|  | GROSS WEIGHT | 20±1KG |  |  |
|  | PACKING | 4PCS/CTN |  |  |

- AE-H2500 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 2500W |  |  |
|  | SURGE POWER | 5000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 370*150*152mm |  |  |
|  | INNER BOX DIMENSION | 530*190*190mm |  |  |
|  | CARTON DIMENSION | 540*290*510mm |  |  |
|  | NET WEIGHT | 7±0.5KG |  |  |
|  | GROSS WEIGHT | 15±1KG |  |  |
|  | PACKING | 2PCS/CTN |  |  |

- AE-H3000 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 3000W |  |  |
|  | SURGE POWER | 6000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 370*150*152mm |  |  |
|  | INNER BOX DIMENSION | 530*190*190mm |  |  |
|  | CARTON DIMENSION | 540*280*520mm |  |  |
|  | NET WEIGHT | 7.5±0.5KG |  |  |
|  | GROSS WEIGHT | 16±1KG |  |  |
|  | PACKING | 2PCS/CTN |  |  |

- AE-H4000 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 4000W |  |  |
|  | SURGE POWER | 8000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 370*150*152mm |  |  |
|  | INNER BOX DIMENSION | 530*190*190mm |  |  |
|  | CARTON DIMENSION | 540*280*520mm |  |  |
|  | NET WEIGHT | 7.5±0.5KG |  |  |
|  | GROSS WEIGHT | 16±1KG |  |  |
|  | PACKING | 2PCS/CTN |  |  |

- AE-H5000 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 5000W |  |  |
|  | SURGE POWER | 10000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 560*240*150mm |  |  |
|  | INNER BOX DIMENSION | 720*335*235mm |  |  |
|  | CARTON DIMENSION | 730*355*265mm |  |  |
|  | NET WEIGHT | 15±0.2KG |  |  |
|  | GROSS WEIGHT | 16±1KG |  |  |
|  | PACKING | 1PCS/CTN |  |  |

- AE-H6000 High frequency pure sine wave inverter

1) Product picture:

2)Technical parameters:

| OUTPUT | OUTPUT VOLT | 100/110/120/220/230/240VAC |  |  |
| --- | --- | --- | --- | --- |
|  | CONTINUOUS POWER | 6000W |  |  |
|  | SURGE POWER | 12000W |  |  |
|  | WAVEFORM | PURE SINE WAVE |  |  |
|  | FREQUENCY | 50Hz/60Hz |  |  |
|  | AC REGULATION | low & high 10% |  |  |
|  | USB CHARGER | 5V |  |  |
|  | LED INDICATOR | LED DISPLAY POWER & FAIL STATUS |  |  |
| INPUT | NO LOAD CURRENT DRAW | 0.2A～0.3A | 0.15A～0.2A | 0.1A～0.15A |
|  | DC VOLTAGE | 10-15VDC | 20-30VDC | 40-61VDC |
|  | VOLTAGE RANGE | 12VDC | 24VDC | 48VDC |
|  | FUSE | 35A | 20A | 15A |
| PROTECTION | BATTERY LOW SHUTDOWN | ±10V |  |  |
|  | OVER TEMPERATURE | ±70℃ |  |  |
|  | BATTERY LOW ALARM | ±10.5V |  |  |
|  | OUTPUT SHORT | SHUT OFF OUTPUT VOLTAGE,RE-POWER ON TO RECOVER |  |  |
|  | OVER LOAD | 120% |  |  |
|  | OVER VOLTAGE | ±15V | ±30V | ±61V |
| OTHER | DIMENSION(L*W*H) | 560*240*150mm |  |  |
|  | INNER BOX DIMENSION | 720*335*235mm |  |  |
|  | CARTON DIMENSION | 730*355*265mm |  |  |
|  | NET WEIGHT | 15±0.2KG |  |  |
|  | GROSS WEIGHT | 16±1KG |  |  |
|  | PACKING | 1PCS/CTN |  |  |

- Address and contract ：

Address: Room 602, Building A Anhui Circular Economy Technology academy of               Engineering,

Changjiang West Road 2221, Hefei ,Anhui,China

Tel：

Fax：

Post code：

E-mail: atone@atoneenergy.com

Website：www.atoneenergy.com